[DISCUSSION DRAFT]

119TH CONGRESS

2d Session

# H.   R.   ll

To amend title 49, United States Code, regarding the authority of the Na-tional Highway Traffic Safety Administration over vehicles with auto-mated driving systems, to provide safety measures for such vehicles, and for other purposes.

IN THE HOUSE OF REPRESENTATIVES

Mr. LATTA introduced the following bill; which was referred to the Committee on llllllllllllll

A  BILL

To amend title 49, United States Code, regarding the author-ity of the National Highway Traffic Safety Administra-tion over vehicles with automated driving systems, to provide safety measures for such vehicles, and for other purposes.

- Be it enacted by the Senate and House of Representa-

- tives of the United States of America in Congress assembled,

- SECTION 1. SHORT TITLE; TABLE OF CONTENTS.

- (a) SHORT  TITLE.—This Act may be cited as the

- ‘‘Safely Ensuring Lives Future Deployment and Research

- In Vehicle Evolution Act of 2026’’ or the ‘‘SELF DRIVE

- Act of 2026’’.

- (b) TABLE OF CONTENTS.—The table of contents for

- this Act is as follows:

Sec. 1. Short title; table of contents. Sec. 2. Purpose.

Sec. 3. Updated or new motor vehicle safety requirements for vehicles with automated driving systems.

Sec. 4. Motor vehicle testing or evaluation. Sec. 5. Make inoperative.

Sec. 6. Protecting the security of connected vehicles. Sec. 7. Definitions.

- SEC. 2. PURPOSE.

- The purpose of this Act is to ensure continued United

- States leadership in the global automotive and autono-

- mous driving sector, improve road safety, mobility, and ac-

- cessibility, and create American jobs by creating rules and

- regulations that relate to the design, construction, and

- performance of ADS-equipped vehicles and by encouraging

- the testing and deployment of such vehicles.

- SEC. 3. UPDATED OR NEW MOTOR VEHICLE SAFETY RE-

- QUIREMENTS  FOR  VEHICLES  WITH  AUTO-

- MATED DRIVING SYSTEMS.

- (a) IN GENERAL.—Subchapter II of chapter 301 of

- subtitle VI of title 49, United States Code, is amended

- by adding at the end the following:

- ‘‘§ 30130.  Motor  vehicle  safety  standards  for  auto-

- mated driving systems

- ‘‘(a) DEFINITIONS.—

  - ‘‘(1) IN GENERAL.—Except as provided in para-

  - graph	(2),	in	this	section	and	sections	30122,

  - 30131, 30132, and 30133:

  - ‘‘(A)	ADS-DEDICATED	VEHICLE.—The

  - term ‘ADS-dedicated vehicle’ means an ADS-

  - equipped vehicle designed to be operated exclu-

  - sively by a Level 4 or Level 5 automated driv-

  - ing system in driverless operation within the

  - operational design domain of the vehicle.

  - ‘‘(B) ADS-EQUIPPED VEHICLE.—The term

  - ‘ADS-equipped vehicle’ means any motor vehicle

  - equipped with an automated driving system.

  - ‘‘(C)	Automated	driving	system;

  - ADS.—

  - ‘‘(i) IN  GENERAL.—The terms ‘auto-

  - mated driving system’ and ‘ADS’—

  - ‘‘(I) mean the hardware and soft-

  - ware that are collectively capable of

  - performing the entire dynamic driving

  - task on a sustained basis, regardless

  - of whether such system is limited to a

  - specific	operational	design	domain;

  - and

    - ‘‘(II) include only a system that

    - meets the definition of Level 3, Level

    - 4, or Level 5 automation.

    - ø‘‘(ii)  APPLICATION.—The  automated

    - driving system and ADS shall be consid-

    - ered to be the driver of a vehicle when the

    - ADS is engaged.¿

    - ‘‘(D) DDT FALLBACK.—The term ‘DDT

    - fallback’ means, after a DDT performance-rel-

    - evant system failure or upon the exit of an

    - ADS-equipped vehicle from the operational de-

    - sign domain of that vehicle—

    - ‘‘(i) the response by the in-vehicle fall-

    - back ready user to perform the DDT to

    - achieve an MRC; or

    - ‘‘(ii) the response by an automated

    - driving system to achieve an MRC.

    - ‘‘(E) DYNAMIC  DRIVING  TASK; DDT.—The

    - terms ‘dynamic driving task’ and ‘DDT’—

    - ‘‘(i) mean each real-time operational

    - and tactical function required to operate a

    - vehicle in on-road traffic;

    - ‘‘(ii) exclude any strategic function,

    - such as trip scheduling and selection of a

    - destination or waypoint; and

      - ‘‘(iii) include without limitation—

      - ‘‘(I) lateral vehicle motion control

      - through steering;

      - ‘‘(II) longitudinal motion control

      - through acceleration and deceleration;

      - ‘‘(III) monitoring the driving en-

      - vironment through object and event

      - detection,	recognition,	classification,

      - and response preparation;

      - ‘‘(IV) object and event response

      - execution;

      - ‘‘(V) maneuver planning; and

      - ‘‘(VI)	enhancing	conspicuity

      - through lighting, sounding the horn,

      - signaling,	gesturing,	or	other	indi-

      - cator.

      - ‘‘(F)  FALLBACK-READY  USER.—The  term

      - ‘fallback-ready user’ means an in-vehicle fall-

      - back-ready user of a vehicle equipped with an

      - engaged Level 3 ADS feature who is properly

      - qualified and able to operate the vehicle and is

      - receptive to ADS-issued requests to intervene

      - and to evident DDT performance-relevant sys-

      - tem failures in the vehicle compelling the user

      - to perform the DDT fallback.

        - ‘‘(G)	In-vehicle	fallback-ready

        - USER.—The	term	‘in-vehicle	fallback-ready

        - user’ means a fallback-ready user of a conven-

        - tional vehicle with an engaged Level 3 ADS fea-

        - ture who is seated in the driver’s seat.

        - ‘‘(H) LEVEL 3.—The term ‘Level 3’ has

        - the meaning given that term in the April 2021

        - edition of the J3016 recommended practice of

        - SAE International, ‘Taxonomy and Definitions

        - for Terms Related to Driving Automation Sys-

        - tems for On-Road Motor Vehicles’.

        - ‘‘(I) LEVEL 4.—The term ‘Level 4’ has the

        - meaning given that term in the April 2021 edi-

        - tion of the J3016 recommended practice of

        - SAE International, ‘Taxonomy and Definitions

        - for Terms Related to Driving Automation Sys-

        - tems for On-Road Motor Vehicles’.

        - ‘‘(J) LEVEL  5.—The term ‘Level 5’ has

        - the meaning given that term in the April 2021

        - edition of the J3016 recommended practice of

        - SAE International, ‘Taxonomy and Definitions

        - for Terms Related to Driving Automation Sys-

        - tems for On-Road Motor Vehicles’.

        - ‘‘(K) Minimal  risk  condition; mrc.—

        - The terms ‘minimal risk condition’ and ‘MRC’

- mean a stable, stopped condition to which an

- in-vehicle fallback-ready user or an ADS may

- bring a vehicle after performing the DDT fall-

- back in order to reduce the risk of a crash when

- a trip cannot or should not be continued.

- ‘‘(L)	Operational	design	domain;

- ODD.—The terms ‘operational design domain’

- and	‘ODD’	mean	the	operating	conditions

- under which an ADS, or feature thereof, is spe-

- cifically designed to function, including any en-

- vironmental, geographical, and time-of-day re-

- striction, or the requisite presence or absence of

- certain traffic or roadway characteristics.

- ‘‘(M)  SYSTEM  FAILURE.—The  term  ‘sys-

- tem failure’ means a malfunction in an ADS,

- other vehicle system, or both that prevents the

- ADS from reliably performing the DDT on a

- sustained basis.

- ‘‘(N)	VULNERABLE	ROAD	USER.—The

- term ‘vulnerable road user’—

- ‘‘(i) means an individual who is not

- an occupant of a motor vehicle with more

- than three wheels; and

- ‘‘(ii) includes a pedestrian, bicyclist,

- motorcyclist,	individual	in	a	wheelchair,

- and a rider or occupant of any other trans-

- port vehicle that is not a motor vehicle,

- such as an all-terrain vehicle or tractor.

- ‘‘(2) Update to definitions.—

- ‘‘(A) Use of sae international’s tax-

- ONOMY AND DEFINITIONS.—The Secretary shall

- use the taxonomy and definitions set forth in

8	April 2021 edition of the J3016 recommended

- practice of SAE International, ‘Taxonomy and

- Definitions for Terms Related to Driving Auto-

- mation Systems for On-Road Motor Vehicles’

- for any term defined in paragraph (1).

- ‘‘(B) REVIEW.—

- ‘‘(i) IN GENERAL.—The Secretary—

- ‘‘(I) shall review the taxonomy

- and definitions for automated driving

- systems	set	forth	by	SAE	Inter-

- national to ensure that such taxonomy

- and definitions are clear and objective;

- and

- ‘‘(II)	may	provide	feedback	to

- SAE International for potential up-

- dates.

- ‘‘(ii) USE OF REVISED STANDARD.—

  - ‘‘(I)	DETERMINATION.—Not

  - later than 120 days after the date on

  - which SAE International revises the

  - taxonomy and definitions referred to

  - in subparagraph (A), the Secretary,

  - after publishing notice of the revision

  - in the Federal Register, shall deter-

  - mine whether to redefine any term de-

  - fined in paragraph (1).

  - ‘‘(II) Effect of decision not

  - TO  ADOPT  THE  REVISED  TAXONOMY

  - AND  DEFINITIONS.—If  the  Secretary

  - decides not to adopt the revised tax-

  - onomy and definitions—

  - ‘‘(aa) the Secretary shall no-

  - tify	SAE	International	of	the

  - Secretary’s decision; and

  - ‘‘(bb)	the	definitions	re-

  - ferred	to	in	subparagraph	(A)

  - shall remain in effect.

  - ‘‘(b)	AUTOMATED	DRIVING	SYSTEM	REQUIRE-

  - MENTS.—

  - ‘‘(1) In general.—

  - ‘‘(A)  SAFETY  CASE  REQUIRED.—A  manu-

  - facturer may not manufacture for sale, sell,

- offer for sale, introduce or deliver for introduc-

- tion into interstate commerce, or import into

- the United States, any automated driving sys-

- tem or ADS-equipped vehicle unless the manu-

- facturer has developed a safety case for the

- automated driving system or ADS-equipped ve-

- hicle that meets the requirements of paragraph

- (3).

- ‘‘(B) Regulations  and  procedures.—

- Consistent with paragraphs (3), (4), and (5),

- the Secretary may prescribe regulations to es-

- tablish—

- ‘‘(i) objective content requirements for

- a safety case; and

- ‘‘(ii) procedures for manufacturers to

- certify the completion of a safety case and

- provide the safety case to the Secretary

- upon request under section 30166.

- ‘‘(C)	Modification	of	ads	com-

- PETENCIES	BY	SECRETARY.—The	Secretary

21	may prescribe motor vehicle safety standards

- that define, modify, or interpret the ADS com-

- petencies described in paragraph (5).

- ‘‘(2) Safety  standard  to  require  safety

- CASE.—

1	‘‘(A) SAFETY  STANDARD.—Not later than

2	September 30, 2027, the Secretary of Trans-

- portation shall issue a final rule that prescribes

- a motor vehicle safety standard under section

- 30111 that requires each manufacturer that

- manufactures for sale, sells, offers for sale, in-

- troduces or delivers for introduction in inter-

- state commerce, or imports into the United

- States an ADS-equipped vehicle or an auto-

- mated driving system intended for installation

- in a vehicle not originally equipped with such

- system, to complete a safety case described in

- paragraph (3) for each ADS version produced

- by a manufacturer that meets the requirements

- of this section and provide the safety case to

- the	Secretary	upon	request	under	section

- 30166.

- ‘‘(B)  DEADLINE.—If  the  Secretary  does

- not meet the deadline described in subpara-

- graph (A), the Secretary shall submit to the

- Committee on Energy and Commerce of the

- House of Representatives and the Committee

- on Commerce, Science, and Transportation of

- the Senate a report that describes the reasons

- why the deadline was not met, including a tar-

- get date for issuing the final rule required pur-

- suant to subparagraph (A).

- ‘‘(C)  SPECIAL  RULE.—Any  motor  vehicle

- safety standard issued under this paragraph su-

- persedes	any	conflicting	requirements	under

- paragraph (1).

- ‘‘(3)	Requirements	for	safety	case.—A

- safety case shall provide claims, supported by argu-

- ments and evidence, that supports the conclusion of

- the manufacturer that the design, construction, and

- performance of the automated driving system or

- ADS-equipped vehicle meets the needs for motor ve-

- hicle safety, including the following:

- ‘‘(A) A description of each hardware and

- software element of the ADS that includes the

- following:

- ‘‘(i) Braking, steering, propulsion, and

- computing capability.

- ‘‘(ii) Redundancies.

- ‘‘(iii) Each capability of the suite of

- sensors of the ADS.

- ‘‘(iv) The integration of the ADS into

- the vehicle platform.

- ‘‘(B) A complete description of the ODD of

- the ADS and how the ADS performs each ele-

- ment of the DDT within the ODD of the ADS.

- ‘‘(C) Each engineering methodology, in-

- cluding each hazard analysis and associated

- verification and validation process, used to de-

- sign and assess the performance of the ADS

- and ensure motor vehicle safety.

- ‘‘(D) The ADS that uses the methodology

- described in subparagraph (C), including com-

- petencies in normal driving and crash avoidance

- capability and the acceptance criteria used by

- the manufacturer in assessing that perform-

- ance.

- ‘‘(E) An explanation of how the ADS an-

- ticipates and responds to potential crashes.

- ‘‘(F)	A	description	of	any	vehicle-inte-

- grated system that provides visual or digital

- hazard alerting to a nearby road user when the

- vehicle executes or enters a minimal risk condi-

- tion, including any system that automatically

- activates	high-conspicuity	lighting	patterns,

- transmits digital hazard messages, or both, to

- connected vehicles and roadway infrastructure.

  - ‘‘(G) An explanation of how the ADS-

  - equipped vehicle meets each competency listed

  - in paragraph (5).

  - ‘‘(4) Optional requirements for a safety

  - CASE.—A safety case completed under this sub-

  - section may include the following:

  - ‘‘(A) A description of the safety manage-

  - ment system of the manufacturer, including or-

  - ganizational roles and responsibilities, and each

  - process	used	to	systematically	address	and

  - audit safety throughout the design, testing, de-

  - ployment, and operation of the automated driv-

  - ing system.

  - ‘‘(B) An explanation of the configuration

  - and change management procedures employed

  - to ensure that any hardware or software up-

  - date, modification, or configuration change is

  - tracked and assessed for safety impacts prior to

  - release, including contingency and rollback pro-

  - cedures where necessary.

  - ‘‘(C) A description of the in-use safety

  - monitoring processes, including safety perform-

  - ance indicators, thresholds for intervention, and

  - procedures	for	investigating	and	addressing

  - safety-relevant incidents and near-misses.

    - ‘‘(D) A description of how the manufac-

    - turer incorporates lessons learned from in-serv-

    - ice operations, including feedback loops from in-

    - cidents and safety-critical scenarios into the de-

    - sign and validation of the ADS, to ensure con-

    - tinuous improvement of motor vehicle safety.

    - ‘‘(5)  ADS  COMPETENCIES.—The  competencies

    - in this paragraph are the following:

    - ‘‘(A) An ADS can perform the entire DDT

    - within the ODD of the ADS and is able to rec-

    - ognize the boundaries of an ODD.

    - ‘‘(B) An ADS can detect and respond ap-

    - propriately to any vulnerable road user likely to

    - be present and in proximity to the ADS in the

    - relevant ODD.

    - ‘‘(C) In the case of a Level 3 ADS, the

    - ability to clearly and unambiguously commu-

    - nicate ADS status and user role and, during

    - any transfer of control to the fallback-ready

    - user, provide sufficient lead time for the fall-

    - back-ready user to safely assume the DDT, and

    - safely maintain vehicle control until the fall-

    - back-user has assumed control and the transfer

    - is completed.

      - ‘‘(D) In the case of a Level 4 and Level 5

      - ADS, the ability to safely achieve an MRC

      - when necessary.

      - ‘‘(E) An ADS can detect the limits of the

      - ODD of the ADS and respond appropriately

      - when one or more conditions of the ODD are

      - no longer met, whether by achieving an MRC

      - or, in the case of a Level 3 ADS, alerting the

      - fallback ready user to intervene and assume

      - control of the DDT.

      - ‘‘(F) An ADS is capable of detecting and

      - responding to any object or event, including

      - emergency vehicles and personnel, and school

      - buses likely to be present and in proximity to

      - the ADS in the relevant ODD and relevant to

      - the driving decisions of the ADS.

      - ‘‘(G) An ADS can comply with applicable

      - local traffic laws and laws relevant to the per-

      - formance of the DDT.

      - ‘‘(H) For a case in which Level 4 or Level

      - 5 ADS is undergoing testing or evaluation or

      - for a case in which a vehicle has human con-

      - trols, Level 4 or Level 5 capabilities, and is

      - being operated manually, the ability to clearly

      - and unambiguously communicate ADS status

- and user role, and safely maintain vehicle con-

- trol until the user has assumed control and the

- transfer is completed.

- ‘‘(I) In the case of a Level 3 ADS, the

- ability to clearly and unambiguously commu-

- nicate ADS status and user role and, during

- any transfer of control to the fallback-ready

- user, provide sufficient lead time for the user to

- safely assume the DDT, and safely maintain ve-

- hicle control until the user has assumed control

- and the transfer is completed.

- ‘‘(J)	A	description	of	the	cybersecurity

- plan that includes a written cybersecurity policy

- with respect to the practices of the manufac-

- turer to detect and respond to cyber attacks,

- unauthorized intrusions, and false vehicle con-

- trol commands that includes—

- ‘‘(i) a process to identify, assess, and

- mitigate reasonably foreseeable cyber risks

- related to motor vehicle safety from cyber

- attacks or unauthorized intrusions, includ-

- ing false and malicious vehicle control com-

- mands; and

- ‘‘(ii) a process to take preventive and

- corrective action to mitigate against rea-

- sonably foreseeable cyber risks related to

- motor vehicle safety in an automated driv-

- ing system or ADS-equipped vehicle, in-

- cluding incident response plans, unauthor-

- ized intrusion detection and prevention sys-

- tems that safeguard key controls, systems,

- and procedures through testing or moni-

- toring, and updates to such process based

- on changed circumstances.

- ‘‘(c) MANUAL CONTROLS.—

- ‘‘(1)	REQUIREMENTS.—A	manufacturer	may

- not sell, offer for sale, introduce or deliver for intro-

- duction into commerce, or import into the United

- States, any ADS-dedicated vehicle that is configured

- or subsequently altered to carry one or more occu-

- pants, unless the vehicle is equipped to—

- ‘‘(A) provide an occupant inside of the ve-

- hicle with the means from within the vehicle to

- command that the ADS safely achieve an MRC;

- and

- ‘‘(B) after the vehicle has achieved an

- MRC, allow the occupant to safely exit the vehi-

- cle.

- ‘‘(2) Exemption from motor vehicle safe-

- TY  STANDARDS.—Provided  the  vehicle  is  equipped

- with the system described in paragraph (1), a motor

- vehicle safety standard issued under section 30111

3	may not be applied to requiring the manufacturer to

- equip the vehicle with manually operated controls

- and equipment intended only to support a human

- driver in an ADS-dedicated vehicle.

- ‘‘(3) A motor vehicle safety standard issued

- under section 30111 may not be applied to require

- a manufacturer of an ADS-dedicated vehicle config-

- ured solely to carry property and not one or more

- occupants to comply with the standard by equipping

- the vehicle with manually operated controls and

- equipment intended only to support a human driver

- in an ADS-dedicated vehicle.

- ‘‘§ 30131. National Automated Vehicle Safety Data Re-

- pository

- ‘‘(a) DEFINITIONS.—In this section:

- ‘‘(1)	COVERED	CRASH.—The	term	‘covered

- crash’ means a crash on a publicly accessible road

- in the United States—

- ‘‘(A) that involves—

- ‘‘(i) a fatality;

- ‘‘(ii) an injury that resulted in a per-

- son being transported from the scene to a

- hospital for medical treatment;

1

2

- user;

4

5

‘‘(iii) an airbag deployment;

‘‘(iv) a strike of a vulnerable road

‘‘(v) a police report; or

‘‘(vi) the towing of a vehicle; and

- ‘‘(B) in which the automated driving sys-

- tem was engaged at any time during the period

- from 30 seconds immediately prior to the com-

- mencement of the crash through the conclusion

- of the crash.

- ‘‘(2)	REPOSITORY.—The	term	‘repository’

- means the National Automated Vehicle Safety Data

- Repository established pursuant to subsection (b).

- ‘‘(3)	State	transportation	regulatory

- AGENCY.—The term ‘State transportation regulatory

- agency’ means a Department of Motor Vehicles, De-

- partment of Transportation, or equivalent State or

- local agency.

- ‘‘(b) RULEMAKING REQUIRED.—Not later than Sep-

- tember 30, 2026, the Secretary of Transportation shall

- issue a final rule that includes the following:

- ‘‘(1) The establishment of a National Auto-

- mated Vehicle Safety Data Repository to receive and

- store data from a covered crash described in para-

- graph (2) from manufacturers of automated driving

- systems and from manufacturers of ADS-equipped

- vehicles.

- ‘‘(2) A requirement, stated in objective terms,

- for such manufacturers to report to the Secretary

- for inclusion in the repository, within 20 days after

- the date on which a covered crash occurs or 7 days

- after notice of such crash, whichever is later, speci-

- fied information known to the manufacturer con-

- cerning the crash including the time and location,

- the vehicles involved, whether vulnerable road users

- were involved, each injury that resulted from the

- crash, whether the vehicle had engaged its DDT-fall-

- back	procedures,	whether	local	authorities	inves-

- tigated the crash, a summary of the circumstances

- surrounding the crash, and if the vehicle was oper-

- ating outside of the ODD.

- ‘‘(3) A requirement for such manufacturers to

- submit to the Secretary on a quarterly basis, for the

- previous quarter, a report on the total miles traveled

- with the ADS engaged of the ADS-equipped vehicles

- of the manufacturer, including the total miles trav-

- eled	in	revenue	service	during	commercial	oper-

- ations.

- ‘‘(4) A process for a relevant State or local

- transportation regulatory agency to access data from

- a covered crash described in paragraph (2) in a

- timely manner.

- ‘‘(5) A requirement that—

- ‘‘(A) any information shared with a rel-

- evant State transportation regulatory agency is

- protected by the confidential business informa-

- tion protections and procedures established in

- part 512 of title 49, Code of Federal Regula-

- tions, and is exempt from disclosure under sec-

- tions 552(b)(3) and section 552a of title 5; and

- ‘‘(B)  any  such  relevant  State  transpor-

- tation regulatory agency is subject to the same

- confidential	business	information	protections

- and procedures established in such part 512.

- ‘‘(6) The establishment of a system to publicly

- share the data collected for the repository, subject to

- the confidential business information protections and

- procedures established in part 512 of title 49, Code

- of Federal Regulations.

- ‘‘(7)	A	requirement	to	minimize	duplicating

- data from a covered crash described in paragraph

- (2) and any other publicly reported data in the re-

- pository.

- ‘‘(8) A requirement for a manufacturer to re-

- port whether an ADS-equipped vehicle involved in a

- covered crash was authorized to operate pursuant to

- section 30112(b)(10).

- ‘‘(c)	SUNSET	FOR	QUARTERLY	MILEAGE	RE-

- PORTS.—

- ‘‘(1) TOTAL  MILES  TRAVELED.—Except as pro-

- vided in paragraph (2), the requirement described in

- subsection (b)(3) shall no longer apply on and after

- the date that is 5 years after the date on which a

- final rule is issued pursuant to subsection (b).

- ‘‘(2)  Extension  of  time  period  for  re-

- PORTS.—Notwithstanding paragraph (1), the Sec-

- retary may extend the requirement described in sub-

- section (b)(3) for two additional years after the expi-

- ration of the requirement described in paragraph (1)

- if the Secretary determines an extension is necessary

- to meet the need for motor vehicle safety.

- ‘‘(d) REPEAL.—The applicability of Standing General

- Order 2021–01 shall no longer apply to ADS-equipped ve-

- hicles on the compliance date of the final rule issued pur-

- suant to subsection (b).

- ‘‘(e) EFFECTIVE DATE.—This section shall take ef-

- fect on September 30, 2026.’’.

- (b) TECHNICAL AND CONFORMING AMENDMENT.—

- The table of sections for subchapter II of chapter 301 of

- subtitle VI of title 49, United States Code, is amended

- by adding at the end the following:

30130. Motor vehicle safety standards for automated driving systems. 30131. National Automated Vehicle Safety Data Repository.

- (c)	ENFORCEMENT	AUTHORITY.—Section

- 30165(a)(1) of title 49, United States Code, is amended

- by inserting ‘‘30130, or 30131’’ after ‘‘30127,’’.

- (d)	RELATIONSHIP	TO	OTHER	LAWS.—Section

- 30103(b) is amended by adding at the end the following:

- ‘‘(3) Federal  preemption  for  automated

- DRIVING SYSTEMS.—

- ‘‘(A)  IN  GENERAL.—A  State,  or  political

- subdivision of a State, may not maintain, en-

- force, prescribe, or continue in effect any law,

- rule, regulation, requirement, standard, or other

- provision having the force and effect of law of

- the State, or political subdivision of the State,

- that—

- ‘‘(i) prohibits in whole or in part a

- manufacturer from manufacturing for sale,

- selling, offering for sale, introducing or de-

- livering	for	introduction	into	interstate

- commerce, or importing into the United

- States, any automated driving system or

- ADS-equipped vehicle if the manufacturer

- develops a safety case described in section

- 30130(b)(3) for the automated driving sys-

- tem or ADS-equipped vehicle; or

- ‘‘(ii) requires manufacturers of auto-

- mated driving systems and manufacturers

- of ADS-equipped vehicles to report covered

- crash data to a State or a political subdivi-

- sion of a State.

- ‘‘(B) EXCEPTIONS.—Notwithstanding sub-

- paragraph (A)—

- ‘‘(i) a State may enforce a law or re-

- quirement that is identical to a law or re-

- quirement prescribed under subparagraphs

- (A) or (B) of section 30130(b)(1) or sec-

- tion 30131; and

- ‘‘(ii) compliance with subparagraphs

- (A) or (B) of section 30130(b)(1) or sec-

- tion 30131 does not exempt a person from

- liability at common law.’’.

- SEC. 4. MOTOR VEHICLE TESTING OR EVALUATION.

- Section 30112 of title 49, United States Code, is

- amended—

- (1) in subsection (b)(10)—

- (A) in subparagraph (C), by striking the

- period at the end and inserting ‘‘; or’’;

  - (B) by redesignating subparagraphs (A)

  - through (C) as clauses (i) through (iii), respec-

  - tively, and moving their margins 2 ems to the

  - right;

  - (C) by striking ‘‘evaluation by a manufac-

  - turer that agrees not to sell or offer for sale the

  - motor vehicle at the conclusion of the testing or

  - evaluation and that prior to the date of enact-

  - ment of this paragraph’’ and inserting the fol-

  - lowing: ‘‘evaluation by—

  - ‘‘(A) a manufacturer that agrees not to sell

  - or lease or offer for sale or lease the motor ve-

  - hicle used for testing or evaluation at the con-

  - clusion of the testing or evaluation and that—

  - ’’; and

  - (D) by adding at the end the following:

  - ‘‘(B) a manufacturer of automated driving

  - systems	or	ADS-equipped	vehicles	(as	such

  - terms are defined in section 30130(a)) or any

  - component of such automated driving systems,

  - that agrees not to sell or lease or offer for sale

  - or lease the automated driving systems, auto-

  - mated driving system-equipped vehicles, or com-

  - ponents of automated driving systems used for

- testing or evaluation at the conclusion of the

- testing or evaluation.’’; and

- (2) by adding at the end the following:

- ‘‘(c)	COMMERCIAL	OPERATIONS	PERMITTED	FOR

- Testing or Evaluation.—

- ‘‘(1) Subject to paragraph (2), the Secretary

7	may authorize a manufacturer that conducts testing

- or evaluation under subsection (b)(10) of section

- 30112 of title 49, United States Code, to undertake

- limited commercial operations, including the car-

- rying of members of the public as passengers and

- transporting freight as part of the testing or evalua-

- tion.

- ‘‘(2) Any evaluation by the Secretary shall con-

- sider the manufacturer’s operational use case, the

- objectives of the manufacturer for the testing or

- evaluation, and be subject to limitations established

- by the Secretary based on the operational use case

- to ensure that the testing or evaluation under this

- subsection does not constitute de facto deployment

- for non-compliant vehicles, that may include—

- ‘‘(A) reasonable limitations on a jurisdic-

- tion-by-jurisdiction basis, on the number of par-

- ticipating vehicles, mileage, revenue generation,

- or duration of deployment; and

  - ‘‘(B) any additional oversight and trans-

  - parency requirements determined by the Sec-

  - retary and is consistent with Secretary’s exist-

  - ing authority in chapter 301, title 49.’’.

  - SEC. 5. MAKE INOPERATIVE.

  - Section 30122(b) of title 49, United States Code, is

  - amended—

  - (1) by inserting ‘‘(1) IN GENERAL.’’ before ‘‘A

  - manufacturer’’; and

  - (2) by adding at the end, the following:

  - ‘‘(2)	EXCEPTION.—Paragraph	(1)	does	not

  - apply in a case in which a manufacturer inten-

  - tionally causes a device or element of a design relat-

  - ing to the performance of the dynamic driving task

  - by a human driver to be temporarily disabled or al-

  - tered in their functionality for safety reasons during

  - the time that an automated driving system is en-

  - gaged and capable of performing the entire dynamic

  - driving task if the system is also capable of being

  - made operative in a manner that would allow a

  - human driver to rapidly regain control over the dy-

  - namic driving task.

  - ‘‘(3) Compliance  with  section  30130 re-

  - QUIRED FOR EXCEPTION.—Any manufacturer that

- claims the exception described in paragraph (2) shall

- comply with section 30130(b).’’.

- SEC. 6. PROTECTING THE SECURITY OF CONNECTED VEHI-

- CLES.

- (a) REVIEW.—Not later than one year after the date

- of the enactment of this Act, the Secretary of Commerce,

- in consultation with the head of any relevant Federal

- agency, shall review the implementation of the final rule

- Securing the Information and Communications Tech-

- nology and Services Supply Chain: Connected Vehicles (90

- Fed. Reg. 5360).

- (b) BRIEFING.—No later than 60 days after the com-

- pletion of the review described in subsection (a), the Sec-

- retary of Commerce shall provide a briefing on implemen-

- tation of the final rule for the Committees on Energy and

- Commerce and Foreign Affairs of the House of Represent-

- atives and the Committees on Commerce, Science, and

- Transportation and Foreign Relations of the Senate.

- SEC. 7. DEFINITIONS.

- In this Act:

- (1)	ADS;	ads-dedicated	vehicle;	ads-

- EQUIPPED VEHICLE; AUTOMATED DRIVING SYSTEM;

- DYNAMIC  DRIVING  TASK; LEVEL  4; LEVEL  5.—The

- terms	‘‘ADS’’,	‘‘ADS-dedicated	vehicle’’,	‘‘ADS-

- equipped vehicle’’, ‘‘automated driving system’’, ‘‘dy-

- namic driving task’’, ‘‘Level 4’’, and ‘‘Level 5’’ have

- the meaning given those terms in section 30130 of

- title 49, United States Code, as added by section 3.

- (2) COVERED CRASH; REPOSITORY.—The terms

- ‘‘covered crash’’ and ‘‘repository’’ have the meanings

- given those terms in section 30131(a), as added by

- section 3(a).

- (3) FOREIGN  ADVERSARY.—The term ‘‘foreign

- adversary’’ means any foreign government or foreign

- non-government person listed in section 791.4(a) of

- title 15, Code of Federal Regulations.

- (4) MOTOR VEHICLE.—The term ‘‘motor vehi-

- cle’’ has the meaning given that term in section

- 30102 of title 49, United States Code.

- (5) SECRETARY.—The term ‘‘Secretary’’ means

- the Secretary of Transportation.

- (6) STATE.—The term ‘‘State’’ means a State

- of the United States, the District of Columbia, Puer-

- to	Rico,	the	Northern	Mariana	Islands,	Guam,

- American Samoa, and the Virgin Islands.